# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply and setup of pilot wild fire video surveillance system in Šibenik-Knin County

Publication reference: HOLISTIC-FB3-12/16

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

IMPORTANT INFORMATION

DESCRIPTION OF THE EQUIPMENT AND REQUESTED AUXILLIARY SERVICES

Šibenik-Knin County is a partner in the strategic project Adriatic Holistic Forest Fire Protection project (HOLISTIC), financed through IPA Adriatic CBC programme. Subject of this procurement contract is procurement of the HOLISTIC pilot wildfire surveillance system for Šibenik- Knin County, to be tested and operated along the period of 5 years as one of the key outcomes within the HOLISTIC project.

Holistic pilot wildfire surveillance system for Šibenik –Knin County is composed of several technical components to be installed at three pre-determined locations for video surveillance spots covering large surface of the southern part of the Šibenik-Knin County and Central data processing and monitoring center to be placed in the County Firefighting operations Center in Šibenik (ŽVOC). Locations for placement of video surveillance equipment have been determined by the technical synthesis report provided by the expert team of the Faculty of electrical engineering, mechanical engineering and naval architecture (FESB), Split.

This contract foresees procurement of overall needed IT equipment for monitoring and surveillance and related services and encompasses procurement, delivery and installation at selected locations, as well as related services of attuning and testing the system and provision of training for the ŽVOC personnel that will be in charge for further management and use of the system. Once installed and accepted by the Contracting Authority, system shall function for at least five years (60 months). In that respect, the Tenderer shall keep in mind to provide all necessary backup support and services for functional use of the system after testing and the Provisional Acceptance, over the 60 months within the required parameters and functionality of the system and its components. Respectively, financial offer for services (i.e. explained under point 9) shall be clearly expressed in unit value (month) and eventual extension over 12 months shall be subject of budget availability and sources of funding. Additional requirements for system guarantees and servicing follow up is provided in technical specifications ad point 15.

Holistic wildfire surveillance system is composed of the following equipment groups:

A: SURVEILLANCE EQUIPMENT AT 3 SURVEILLANCE SPOTS AND COUNTY FIREFIGHTING OPERATIONS CENTER with all sub-items specified in the technical specifications

B: SOFTWARE SYSTEM FOR WILDFIRE MONITORING AND RECOGNITION with all sub-items specified in the technical specifications

C: OTHER with all sub-items specified in the technical specifications

- This group entails provision of masts or places for placement of video and other IT equipment at three locations, ensuring connectivity and data transfer in given parameters with ŽVOC from the surveillance spots in all in accordance with Technical report (Annex 1).

- Training of ŽVOC personnel on management and use of the system

- After sales service for system functional operation on annual basis for next five years

More detailed instructions on system functioning and parameters of each location are given in Technical report prepared by FESB (Annex 1 to TS), including data related to coverage, rationale for selection and existing infrastructure at each location.

Please note that tenderer is obliged to provide copy of documentary proof for use of the basic infrastructure (mast/ solid object or else with other requirements explained under point below) for specific three observation spots/locations for period of 60 months, either in form of the contract or statement issued by the infrastructure owner or entitled user. These proofs shall be submitted along with the Technical specifications as supporting documents!

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| A: SURVEILLANCE EQUIPMENT AT 3 SURVEILLANCE SPOTS AND COUNTY FIREFIGHTING OPERATIONS CENTER |  |  |  |  |
|  | Monitoring/surveillance equipment |  |  |  |
| 1. | Monitoring/surveillance video camera (6 pcs) - High-resolution PTZ IP dome video camera steered in azimuth, elevation and zoom, with minimal image resolution 1920x1080 px (Full HD) configurable on lower resolution up to 320 x 180 px - Automatic switching from day mode (colour camera sensitive in the visible spectrum) to night mode (camera is sensitive in the near infrared part of the spectrum) - Optical zoom of at least 30x and 12x digital zoom, - Variable field of view horizontal from > = 600 to <= 30, - Minimum sensitivity in colour (day) mode: 0.3 lux at 30 IRE F1.6; black and white: 0:03 lux at 30 IRE F1.6, - Rotation in azimuth 3600 (continuous) turning in elevation at least 2200, variable speed of 0.05 0/s up to at least 400 0/sec, - Set the reference north, - The possibility of setting a minimum of 50 pre-set positions with different azimuth, elevation and zoom settings, - The camera should be integrated with video Web server with HTTP API functions support for camera integration with software. HTTP API functions support: - Video compression formats motion JPEG and H.264, - Adjustment of individual video 'streams' according to resolution, speed (fps) and the degree of compression, - Access to the current image in JPEG format, - Relative camera control in azimuth and elevation (relative shift from the current position to x0 and in azimuth and y0 in elevation), - Absolute camera control in azimuth, elevation and zoom (positioning of camera according to reference north in azimuth position x0, the elevation in position y0, and the zoom in zoom xN), - Continuous camera movement in azimuth and elevation in any direction with variable speed (virtual joystick), - Set up, modify and delete all the present positions, - Feedback information from the camera about current azimuth, elevation and zoom; - Watertight housing, degree of protection IP66 - Working environment: temperature from -20 °C to +50 °C / 90% humidity - Power on the principle of PoE (Power over Ethernet). Unit power less than 80 W - Camera weight up to 5 kg Warranty: 1 year |  |  |  |
| 2. | IT Support (network, link, protection) equipment at surveillance spots (3 pcs) - Network switch: A minimum of 8 ports 100Base-TX, of which at least 4 ports PoE 802.3af - Remote device with an on/off switch for cameras with a minimum of 8 relay inputs/outputs, built-in Web server that includes HTTP API for software integration with the function of remote switching on/off of individual relay channels - Overvoltage protection of all components - Manual on/off switch for cameras - Fuses - Protection cabinet class IP66 of sufficient dimension to hold all components Warranty: 1 year NOTE: The supplier must provide listed equipment for all 3 surveillance locations where video cameras are placed. It should list eventually additional equipment needed for functional data transfer. |  |  |  |
|  | IT Equipment for County Firefighting operations Center in Šibenik (ŽVOC) |  |  |  |
| 3 | Server (QTY 1 piece) - CPU: 2x quad core, frequency 3,6 GHz, benchmark test score 9000 - RAM: 36GB, DDR4, expandable to min 72 GB - Hard drive: 5 x 600GB 6G 2,5'' 10000 rpm - Controller with support for SATA / SAS disks and RAID levels 0, 1, 10, 5, 50, 6, 60 - Network card: adapter with 2 ports 10/100/1000 Mbps - OS adequate for optimized functioning of the software system under point 8 TS Warranty: 3 years |  |  |  |
| 4 | Server rack cabinet (QTY 1 piece) - Fit for the server under point 3, equipped with cooling device and other relevant devices Warranty: 3 years |  |  |  |
| 5 | Desk computer (QTY 1 piece) - CPU: dual core, frequency 3,6 GHz, benchmark score 2000 - RAM : 8GB, DDR3 - Hard drive: 1 TB, SATA HD 7200 rpm - Graphic card: support for 2 full HD monitors (1920 x 1080 px), 2xHDMI connectors - Network Card 10/100/1000 Mbps Ethernet - OS adequate for optimized functioning of the software system under point 8 TS - Keyboard, HR layout - Mouse Warranty: 3 years |  |  |  |
| 6 | Monitor /TV(QTY 2 pcs) - LED technology, Full HD (1080p), size 40'' - Response time: 8ms - Refreshment frequency min 60 Hz - Connectivity: HDMI, USB 3.0, VGA - Wall mount holder Warranty: 3 years |  |  |  |
| 7 | UPS (QTY as per system needs) - Provides full operability of the server centre for 30 minutes after power shortage (server, PC and monitors) Warranty: 1 year NOTE: Tenderer is obliged to provide calculation of needed capacity for the UPS as proof for offered device TS. |  |  |  |
| B: SOFTWARE SYSTEM FOR WILDFIRE MONITORING AND RECOGNITION |  |  |  |  |
| 8. | Wildfire monitoring and surveillance Software system (QTY 1) - System functionalities compliant to rules and regulations of the document “Pravilnik za zaštitu šuma od požara” (NN 033/2014) - Mode 1: monitoring mode with automatic detection of the visible signs of smoke and fire with the following functionalities: - Automatic detection of the visible signs of smoke and fire in real time, 24 hours a day, - Notifying fire surveillance operator of visible signs of smoke and fire with audio and visual signals, - Ability to customize the parameters (sensitivity) for automatic detection in accordance to atmospheric and other conditions, independently for each camera and each pre-set position, - Display of orientation and field of view of the cameras on a geo-referenced map, - Storage of all incoming images and all alarms (detected visible signs of smoke and fire), - Ability to send data about confirmed alarm to the Web service of the external GIS application “AdriaFireGIS” (please refer to Technical report, Annex 1) ; - Mode 2: surveillance mode with the following functionalities: - ability to manually control cameras per azimuth, elevation and zoom, - ability to quickly set up a camera in a pre-defined preset positions, - Display of orientation and field of view of the cameras on a geo-referenced map - storage of all input images. - Ability to review archives of all incoming images for the last 168 hours, filtered by date and time, - Ability to review the archive of all the alarm images for the current fire season, - Ability for passive viewing of incoming and / or processed images without the possibility of controlling the system, - Multiple user accounts with different levels of authorization (administrator, operator and viewer) - Ability to adjust system parameters at different authorization levels - Recording, listing and view of each user activity history - Remote access using standard web browsers and following OS: Windows, MacOS, Linux - Virtual Private Network (VPN) encompasses entire surveillance system with firewall protection* NOTE: supplier is obliged to ensure provision of configuration files for remote access to the system for external users. List of users will be provided to the contractor during the implementation of the task. Warranty: 1 year |  |  |  |
| C: OTHER |  |  |  |  |
| 9 | Ensure basic infrastructure for placement and installation of the monitoring/surveillance equipment described under point 1 and 2 |  |  |  |
| 9.1 | Provision of monitoring location spot at location Kamenar hill on monthly basis QTY: 60 months Geo coordinates: 43°44'12.77"N 15°54'58.23"E Ground altitude 256m Cameras installment altituted 266m More information on location available in Technical report (Annex 1) - Providing placement of equipment at existing mast(s) or other stable elevated structures adequate for mounting of the surveillance equipment ensuring visible area as per requested parameters - Communication links for data transfer between the surveillance spot and the surveillance centre - Uninterruptible power supply for equipment functioning - Lightning and overvoltage protection |  |  |  |
| 9.2 | Provision of monitoring location spot at location Dubravice on monthly basis QTY: 60 months Geo coordinates: 43°51'27.30"N 15°56'21.58"E Ground altitude 219m Cameras installment altituted 229m More information on location available in Technical report (Annex 1) - Providing placement of equipment at existing mast(s) or other stable elevated structures adequate for mounting of the surveillance equipment ensuring visible area as per requested parameters - Communication links for data transfer between the surveillance spot and the surveillance centre - Uninterruptible power supply for equipment functioning - Lightning and overvoltage protection |  |  |  |
| 9.3 | Provision of monitoring location spot at location Srima on monthly basis QTY: 60 months Geo coordinates: 43°45'8.21"N 15°48'24.43"E Ground altitude 18m Cameras installment altituted 28m More information on location available in Technical report (Annex 1) - Providing placement of equipment at existing mast(s) or other stable elevated structures adequate for mounting of the surveillance equipment ensuring visible area as per requested parameters - Communication links for data transfer between the surveillance spot and the surveillance centre - Uninterruptible power supply for equipment functioning - Lightning and overvoltage protection |  |  |  |
| 10. | Provision of adequate connectivity on monthly basis (QTY: 60 months) ŽVOC server - Communications link of flat type with minimum speed of 12 Mb/s upload, 12 Mb/s download while ensuring IP address for server inside VPN system. Monitoring locations - Communications link of flat type with minimum speed of 4 Mb/s upload, 200 kb/s download while ensuring IP address for each camera and cameras on/off device inside VPN system. |  |  |  |
| 11. | Installation of the equipment under points 1 and 2 on 3 locations defined under point 9. - Mounting of 2 cameras and support equipment on chosen masts/objects for each surveillance spot - Ensure power supply for functional work of the surveillance equipment and communications according to specifications (ad point 1 and 2). In case of power shortage, equipment shall run for at least 30 minutes after. - Establish two way communication with the server centre and configure the network in accordance with specification under point 10. - Ensure full functionality of the surveillance spots NOTE: If besides the listed equipment additional equipment is needed to ensure functionality and security of the system in accordance to the highest standards, the supplier is obliged to specify it and include it in its offer. |  |  |  |
| 12. | Installation and integration of equipment under points 3. – 7. in “ŽVOC” Šibenik and its integration with existing IT equipment and infrastructure in ŽVOC NOTE: If besides the listed equipment additional equipment is need to ensure functionality and security of the system in accordance to the highest standards, the supplier is obliged to specify it and include it in its offer. |  |  |  |
| 13. | Integration, initiation, testing and optimization of the overall monitoring and surveillance system Tenderer shall propose procedure for testing of the overall system and implement it in coordination with the Contracting Authority and ŽVOC personnel. |  |  |  |
| 14. | Training for use of the surveillance system Upon completed final attuning and system test, an in-house training for 6 ŽVOC staff in charge for further management and operation of the system shall be held in ŽVOC premises in Šibenik. Trained personnel shall be provided with manuals and guidelines for system use printed in Croatian language. |  |  |  |
| 15. | After sales services: Regular maintenance and ad-hoc maintenance of the overall system with maximum 24 hours response time for all components of the system based on annual service costs |  |  |  |